DESIGN NOTES, WIREFRAMES & SYSTEM ARCHITECTURE COMPENDIUM BlueHawk Civic Nexus Autonomous Edge-AI Street Rover (H.A.W.K.) & GIS Urban Governance Grid (BlueBridge) Proactive Emergency Threat Alerting and Sustainable Municipal Action

| THEME 4 Governance & Civic Technology |  | THEME 7 AI, GIS & Emerging Technologies |  | SDGs 3 ¦ 9 ¦ 11 ¦ 16 |
| --- | --- | --- | --- | --- |

| 6 Core sections |  | 10 Figures & diagrams |  | 4 UI wireframes |  | 12 Month roadmap |
| --- | --- | --- | --- | --- | --- | --- |

| Submitted to | MY Bharat Portal: "Upload any other design documentation" (supporting the 7-slide pitch deck) |
| --- | --- |
| Framework | Ministry of Youth Affairs & Sports (MoYAS) and IIT Bombay VBYLD 2027 Framework |
| Team / State | BlueHawk Civic Nexus ¦ Uttar Pradesh |
| Lead Innovator | Aravind K. T., B.Tech Electrical Engineering (Class of 2028), Indian Institute of Technology Kanpur, Hall of Residence 5 ¦ AISHE Code: U-0517 |
| Document | Version 1.0 ¦ 22 October 2026 ¦ Design Notes, Wireframes & System Architecture |

Portfolio: https://sites.google.com/view/aravind-k-t-iitk/

Live App: https://bluehawk-civic-nexus.streamlit.app

GitHub: https://github.com/kt2141/BlueHawk-Civic-Nexus

# Contents

| Section | Title | What it covers |
| --- | --- | --- |
| 1 | Executive Summary & System Vision | The dual crisis, the unified H.A.W.K.-Nexus paradigm, design principles, closed-loop concept |
| 2 | Hardware Engineering & Embedded Specifications | H.A.W.K. platform, power architecture, reference pin map, state machine and C++ logic, evaluation routines, failure-analysis log |
| 3 | Software, AI Classifier & GIS Spatial Grid | Ingestion pipeline, dual-triage routing (Path A / Path B), PostGIS data model, heatmap scoring, feedback cycle, security |
| 4 | UI/UX Wireframes & Dashboard Specifications | Rover telemetry tab, citizen portal, GIS heatmap and triage register, Youth Action Log |
| 5 | Scalability, Feasibility & Deployment Roadmap | 12-month phased blueprint, KPIs, risk register, scaling plan |
| 6 | Innovator Credentials & Verification References | Academic, award and delegation dossier; official links |
| A | Annexure: Document Notes | Reference-design conventions, glossary, AI-use disclosure |

HOW TO READ THIS COMPENDIUM Every section pairs a design decision with its engineering rationale. Figures are provided as diagrams and as text layouts so the content survives plain-text review. Items marked Reference design (pin allocation, firmware, database schema, scoring weights) are the documented baseline architecture; the as-built source is in the public repository.

LIST OF FIGURES

| Fig. | Title | Section |
| --- | --- | --- |
| 1 | Closed-loop operating cycle | 1 |
| 2 | End-to-end layered system architecture | 1 / 3 |
| 3 | H.A.W.K. hardware and power block diagram | 2 |
| 4 | Differential-drive state machine | 2 |
| 5 | Dual-triage decision flow (Path A / Path B) | 3 |
| 6 | Wireframe 1: Rover telemetry & vision feed | 4 |
| 7 | Wireframe 2: Citizen omni-channel reporting portal | 4 |
| 8 | Wireframe 3: GIS ward heatmap & triage register | 4 |
| 9 | Wireframe 4: Youth-Led Dialogue Circles action log | 4 |
| 10 | 12-month deployment roadmap | 5 |

# 1. Executive Summary & System Vision

EXECUTIVE BRIEF BlueHawk Civic Nexus joins two proven building blocks into one accountable civic system: H.A.W.K., a low-cost autonomous street-patrol rover that senses and sees at the edge, and BlueBridge, an AI/NLP classification engine on a PostgreSQL/PostGIS spatial grid that turns every observation into a located, prioritized, trackable action. Youth-Led Peace & Dialogue Circles close the loop on the ground.

## 1.1 The Dual Crisis in Urban & Vulnerable Wards

Urban and semi-urban wards face two problems at once, and treating them separately is why neither gets fixed. One is loud and visible; the other is quiet and structural.

| Dimension | Visible crisis | Invisible crisis |
| --- | --- | --- |
| What it is | Uncollected garbage, drainage blockages, waterlogging | Governance system gaps and inadequate street-level security monitoring in vulnerable corridors |
| How it shows up | On the street, in photos, in complaints | In missing data, unlinked complaint channels, late or no response |
| Who feels it | Residents, sanitation crews, health workers | Residents in vulnerable corridors, police control rooms, ward officials |
| Core failure | Reactive cleanup with no prioritization | No shared, geo-referenced picture; no feedback to the citizen |

THE DWIVIDHTA (DESIGN DUALITY) FRAMEWORK Every alert carries two halves. Technology precision: an AI flag, a GIS coordinate and a timestamp. Community empathy: a Youth-Led Peace & Dialogue Circle that verifies, explains and builds trust. An item is closed only when both halves are satisfied.

## 1.2 The Unified H.A.W.K.-Nexus Paradigm

The paradigm closes the loop between three layers that are usually built in isolation:

- Autonomous edge sensing: the rover patrols managed corridors, detects hazards and threats with on-board and edge-node vision, and tags them with location.

- Cloud GIS routing: BlueBridge classifies each item, indexes it spatially, and routes it automatically: emergencies to the Police Control Room, municipal hazards to the ward heatmap and crews.

- Grassroots civic execution: crews and Youth-Led Dialogue Circles act, verify and report back, refreshing the public dashboard.

Figure 1. Closed-loop operating cycle: detect, classify, locate, route, act, verify.

End-to-end data path in text form:

H.A.W.K. ROVER --------+ +--> PATH A: EMERGENCY THREAT (sensors + edge vision)¦ +-----------+ +----------+ ¦ live alert + target tracking +---->¦ AI / NLP ¦-->¦ PostGIS ¦--+ --> POLICE CONTROL ROOM (PCR) ¦ ¦ TRIAGE ¦ ¦ GRID ¦ ¦ CITIZEN WEB / MOBILE --+ +-----------+ +----------+ +--> PATH B: MUNICIPAL HAZARD (omni-channel intake) ward heatmap + sanitation routes ^ --> CREWS + YOUTH ACTION LOG ¦ ¦ +---- Dialogue Circle verification <-- public dashboard refresh <----+

## 1.3 Design Principles

| Principle | What it means in this design |
| --- | --- |
| Low cost, modular | Arduino / ESP32 class hardware; each module (drive, sensing, vision, uplink) is replaceable on its own. |
| Open source | Python, OpenCV, YOLO, Streamlit and PostgreSQL/PostGIS; no licence barrier for municipalities. |
| Human in the loop | Low-confidence detections go to a review queue; emergency alerts are verified at the PCR before dispatch. |
| Privacy by design | Inference at the edge; only geotagged event records and short evidence clips are stored; role-based access. |
| Community verified | Dialogue Circles verify closure on the ground, so dashboards reflect lived reality. |
| Fail-safe | The rover defaults to STATE_STOP on sensor fault, low battery or thermal cut-off. |

## 1.4 Alignment

| Item | Alignment |
| --- | --- |
| Themes | Theme 4: Governance & Civic Technology ¦ Theme 7: AI, GIS & Emerging Technologies |
| UN SDGs | SDG 3 (health: fewer waste and waterlogging hazards) ¦ SDG 9 (innovation and infrastructure) ¦ SDG 11 (sustainable cities) ¦ SDG 16 (strong, transparent institutions) |
| Programme | Ministry of Youth Affairs & Sports (MoYAS) and IIT Bombay VBYLD 2027 Framework |

# 2. Hardware Engineering & Embedded Specifications

H.A.W.K. module. The hardware foundation is a 4-wheel mobile platform built around an Arduino Uno R3, extended with an edge computer-vision node so that detections happen on the rover instead of in the cloud.

REFERENCE DESIGN NOTICE The pin allocation, firmware listing, power-rail values and test criteria below form the documented reference design for H.A.W.K. They are written to be reproducible. The as-built firmware and wiring are published in the public repository; where the two differ, the repository is authoritative.

## 2.1 Platform Overview

| Chassis | 4-wheel differential-drive mobile platform (left pair / right pair driven together) |
| --- | --- |
| Main controller | Arduino Uno R3 (ATmega328P), runs sensing loop and drive state machine |
| Motor driver | Adafruit Motor Shield (stacked on the Uno) driving 4 DC motors |
| Proximity sensing | HC-SR04 ultrasonic range sensing; dual IR line/proximity sensors for close-range and edge detection |
| Environmental sensing | UV sensor(s) |
| Actuators | Metal-gear servos (sensor scanning / pan) |
| Edge vision | ESP32-CAM or Raspberry Pi Zero 2W running OpenCV + YOLO bounding-box detection; telemetry link to the Uno |
| Power | 11.1 V (3S) Li-Ion battery bank with buck converters for logic, servo and vision rails |

Figure 2. H.A.W.K. hardware and power block diagram.

## 2.2 Mechanical & Electrical Schematic Breakdown

### Reference pin allocation

The Adafruit Motor Shield (V1 / AFMotor) reserves D3, D4, D5, D6, D7, D8, D11 and D12 for motor PWM and its shift-register, and D9/D10 for the two servo headers. The remaining pins are allocated to sensing as follows.

| Function | Arduino pin | Notes |
| --- | --- | --- |
| DC motors M1 to M4 | Shield-managed (D3, D5, D6, D11 PWM; D4, D7, D8, D12 shift register) | M1+M2 = left side, M3+M4 = right side |
| Servo (scan) | D9 (shield SER2) | Attached only while scanning (thermal protection) |
| HC-SR04 TRIG / ECHO | A0 / A1 | Digital use of analog pins; 30 ms echo timeout |
| IR left / right | A2 / A3 | Digital read, LOW = obstacle detected |
| UV sensor | A4 | Analog read, reported in telemetry |
| Battery monitor | A5 | Resistor divider from pack; drives low-voltage fail-safe |
| Vision link (UART) | D2 (RX) / D13 (TX) | SoftwareSerial to ESP32-CAM / Pi Zero 2W |

### Power management

| Rail | Source | Feeds | Protection / design rule |
| --- | --- | --- | --- |
| Battery bus | 11.1 V Li-Ion 3S pack | All rails | Main fuse + master switch; common ground; do not discharge below about 9.0 V (3.0 V per cell) |
| 5 V logic | Buck converter 1 | Arduino, sensors, IR, UV | Decoupling capacitors close to sensors; kept separate from motor noise |
| Servo / vision | Buck converter 2 | Servos, ESP32-CAM / Pi Zero 2W | Dedicated rail so servo surges and camera boot current do not reset the controller |
| Motor supply | Battery bus (fused) | Adafruit Motor Shield | Fuse sized above running current and below wiring limit; flyback handled by driver |

## 2.3 Embedded C++ State Machine Logic

Differential drive is implemented as a four-state machine. Sensing runs every loop iteration; transitions are evaluated from the latest ultrasonic and IR readings.

| State | Entry action | Leaves when |
| --- | --- | --- |
| STATE_STOP | Both sides released | Path clear > FORWARD; else the clearer side > TURN_LEFT / TURN_RIGHT. Held while a fault is active. |
| STATE_FORWARD | Both sides driven forward | Ultrasonic below threshold, or either IR sees an obstacle > STOP |
| STATE_TURN_LEFT | Left side reverse, right side forward | Turn time elapsed > FORWARD (re-evaluate) |
| STATE_TURN_RIGHT | Left side forward, right side reverse | Turn time elapsed > FORWARD (re-evaluate) |

Figure 3. Differential-drive state machine with fail-safe stop.

Reference firmware listing (Arduino C++):

#include <AFMotor.h> // Adafruit Motor Shield V1 library #include <Servo.h> #include <SoftwareSerial.h> // ---- Reference pin allocation (confirm against as-built firmware) ---- const uint8_t PIN_TRIG = A0, PIN_ECHO = A1; // HC-SR04 const uint8_t PIN_IR_L = A2, PIN_IR_R = A3; // dual IR, LOW = obstacle const uint8_t PIN_UV = A4, PIN_BATT = A5; // UV sensor, battery divider SoftwareSerial visionLink(2, 13); // RX, TX to ESP32-CAM / Pi Zero 2W AF_DCMotor m1(1), m2(2), m3(3), m4(4); // left: M1,M2 right: M3,M4 Servo scanServo; // shield SER2 -> D9 enum State { STATE_STOP, STATE_FORWARD, STATE_TURN_LEFT, STATE_TURN_RIGHT }; State state = STATE_STOP; uint32_t stateSince = 0; const uint16_t OBSTACLE_CM = 25; // tunable const uint8_t DRIVE_SPEED = 180; // 0-255 const uint16_t TURN_MS = 450; const uint16_t BATT_MIN_RAW = 600; // set from divider ratio (about 9.0 V on 3S) uint16_t readCm() { digitalWrite(PIN_TRIG, LOW); delayMicroseconds(2); digitalWrite(PIN_TRIG, HIGH); delayMicroseconds(10); digitalWrite(PIN_TRIG, LOW); unsigned long us = pulseIn(PIN_ECHO, HIGH, 30000UL); return us ? us / 58 : 999; // 999 = no echo } void setSide(AF_DCMotor &a, AF_DCMotor &b, int v) { uint8_t d = v > 0 ? FORWARD : (v < 0 ? BACKWARD : RELEASE); a.setSpeed(abs(v)); b.setSpeed(abs(v)); a.run(d); b.run(d); } void drive(int l, int r) { setSide(m1, m2, l); setSide(m3, m4, r); } bool faultActive() { return analogRead(PIN_BATT) < BATT_MIN_RAW; } void enter(State s) { // entry actions state = s; stateSince = millis(); switch (s) { case STATE_STOP: drive(0, 0); break; case STATE_FORWARD: drive(DRIVE_SPEED, DRIVE_SPEED); break; case STATE_TURN_LEFT: drive(-DRIVE_SPEED, DRIVE_SPEED); break; case STATE_TURN_RIGHT: drive(DRIVE_SPEED, -DRIVE_SPEED); break; } } void setup() { pinMode(PIN_TRIG, OUTPUT); pinMode(PIN_ECHO, INPUT); pinMode(PIN_IR_L, INPUT); pinMode(PIN_IR_R, INPUT); Serial.begin(9600); visionLink.begin(9600); enter(STATE_STOP); } void loop() { uint16_t cm = readCm(); bool blockL = digitalRead(PIN_IR_L) == LOW; bool blockR = digitalRead(PIN_IR_R) == LOW; if (faultActive() && state != STATE_STOP) enter(STATE_STOP); // fail-safe switch (state) { case STATE_FORWARD: if (cm < OBSTACLE_CM ¦¦ blockL ¦¦ blockR) enter(STATE_STOP); break; case STATE_STOP: if (faultActive()) break; if (cm >= OBSTACLE_CM && !blockL && !blockR) enter(STATE_FORWARD); else if (!blockL) enter(STATE_TURN_LEFT); else if (!blockR) enter(STATE_TURN_RIGHT); break; case STATE_TURN_LEFT: case STATE_TURN_RIGHT: if (millis() - stateSince > TURN_MS) enter(STATE_FORWARD); break; } visionLink.print(state); visionLink.print(','); visionLink.println(cm); // telemetry delay(30); }

## 2.4 Evaluation Routines

Each routine has a defined procedure and acceptance criterion. Measured values are recorded in the test log kept with the repository.

| Routine | Procedure | Acceptance criterion |
| --- | --- | --- |
| Ultrasonic calibration | Compare readings against a tape-measured target at fixed distances | Readings stable and within the sensor's rated tolerance |
| Obstacle-stop test | Drive at DRIVE_SPEED toward a fixed obstacle | Rover enters STATE_STOP before contact on every run |
| IR edge / proximity test | Present obstacles to the left and right IR sensors separately | Correct side detected; correct turn direction chosen |
| Thermal soak test | Run continuous patrol with scanning servo active; check servo and driver temperature | No thermal shutdown; temperature stabilizes within safe limits |
| Battery cut-off test | Lower supply to the cut-off threshold on a bench supply | Rover enters and holds STATE_STOP |
| Vision accuracy test | Run labelled test frames through the edge detector | Precision and recall recorded per class; thresholds tuned to minimize false emergency alerts |
| Endurance patrol | Repeated managed-corridor patrol over a full battery | Completes patrol without manual intervention; telemetry uplink continuous |

## 2.5 Iterative Engineering & Failure Analysis Log

| # | Problem observed | Root cause | Corrective action | Outcome |
| --- | --- | --- | --- | --- |
| 1 | Heavy mild-steel wheels made the platform sluggish and strained the drive motors | Wheel mass and rotational inertia exceeded the comfortable torque envelope of the small DC gear motors | Replaced with rugged rubber/plastic wheels | Lower motor load, better traction, more responsive turns |
| 2 | Servo wear and heating under continuous load | Servo held position or jittered continuously, sustaining high current and heat | Thermal protection upgrade: detach servo when idle, limit scan duty cycle, dedicated servo rail, cool-down interval | Servo runs cooler; wear reduced; no thermal shutdowns in soak testing |

## 2.6 Key Design Decisions & Trade-offs

| Decision | Choice | Rationale | Trade-off and how it is handled |
| --- | --- | --- | --- |
| Main controller | Arduino Uno R3 | Simple, robust, widely supported; ideal for deterministic sensing and drive control | Limited compute, so vision runs on a separate edge node |
| Vision at the edge | ESP32-CAM / Raspberry Pi Zero 2W | Low cost and power; detections happen on the rover, so only results are uplinked | Small models only; lightweight YOLO configuration and confidence thresholds with human review |
| Drive layout | 4-wheel differential drive | Mechanical simplicity and in-place turns suit narrow corridors | Skid on turns; handled with rubber/plastic wheels and short turn intervals |
| Power | 11.1 V Li-Ion 3S + buck converters | Headroom for motors with standard, available cells; clean separate rails | Needs low-voltage cut-off and fusing, both built into the design |
| Default behaviour | STATE_STOP on any fault | A stopped rover is safe in public space | Patrol pauses until the fault clears or an operator intervenes |

ENGINEERING TAKEAWAY Both failures were resolved by reducing load and managing duty cycle, not by adding cost. That keeps H.A.W.K. inside its low-cost, replicable design envelope.

# 3. Software, AI Classifier & GIS Spatial Grid

BlueBridge module. BlueBridge is the software and governance grid: ingestion, AI/NLP classification, spatial indexing, routing and the dashboards officials and citizens use.

Figure 4. End-to-end layered system architecture (reference design).

## 3.1 Multi-Modal Ingestion Pipeline

| Channel | Content | Pre-processing | Tagged with |
| --- | --- | --- | --- |
| H.A.W.K. telemetry | State, range, IR, UV, battery | Validated and time-stamped on the rover | Rover ID, location, time |
| H.A.W.K. video / frames | Camera frames with bounding boxes | YOLO + OpenCV on the edge node; only detections and short evidence clips uplinked | Location, confidence, class |
| Citizen web portal | Text, photo, location | NLP classification, duplicate check | GPS pin or address, time |
| Citizen mobile app | Text or voice note, photo, auto GPS | Speech-to-text where used, then NLP | GPS pin, time |
| Helpline text (optional) | Short messages | NLP classification; location asked back if missing | Address / landmark |

## 3.2 Dual-Triage Routing Engine

Every ingested item passes through one triage engine that outputs a class, a confidence score and a path.

Figure 5. Dual-triage decision flow. Low-confidence items go to a human review queue.

### Path A: Emergency threats

- Real-time bounding-box detection from the rover's edge vision node.

- Alert routed directly to the Police Control Room (PCR) dashboard with the frame, location, time and active target-tracking.

- PCR operator verifies before dispatch (human in the loop); false positives are logged to retrain the model.

### Path B: Municipal hazards

- Automated PostgreSQL/PostGIS spatial mapping of every garbage, drainage or waterlogging item.

- Green-to-red hotspot visualization per ward, with automatic priority sanitation route generation for crews.

- Status updates flow back to the citizen and to the Youth Action Log.

## 3.3 PostGIS Spatial Data Model

| Table | Purpose | Key fields |
| --- | --- | --- |
| ward | Ward boundary polygons | ward_id, name, geom (MultiPolygon, SRID 4326) |
| event | Every geotagged hazard or threat | source, class, path, confidence, status, ward_id, reported_at, resolved_at, geom (Point) |
| class_weight | Severity weight per class | class, path, weight (tunable by the ward office) |
| rover_telemetry | Rover health and trail | rover_id, state, battery, range, geom, time |
| youth_action | Youth Action Log entries | event_id, circle_id, action, outcome, verified_at |
| app_user | Role-based access | role (citizen, crew, PCR operator, ward officer, admin) |

Reference schema and hotspot query:

CREATE EXTENSION IF NOT EXISTS postgis; CREATE TABLE ward ( ward_id serial PRIMARY KEY, name text NOT NULL, geom geometry(MultiPolygon, 4326) NOT NULL); CREATE TABLE class_weight (class text PRIMARY KEY, path char(1), weight real); CREATE TABLE event ( event_id bigserial PRIMARY KEY, source text CHECK (source IN ('rover','citizen_web','citizen_app')), class text NOT NULL, -- litter, drain_block, waterlogging, person ... path char(1) CHECK (path IN ('A','B')), confidence real, status text DEFAULT 'open', ward_id int REFERENCES ward, reported_at timestamptz DEFAULT now(), resolved_at timestamptz, geom geometry(Point, 4326) NOT NULL); CREATE INDEX event_geom_gix ON event USING GIST (geom); -- Raw hotspot score per ward (open Path B items, older items weigh more) SELECT w.ward_id, w.name, SUM(c.weight * (1 + EXTRACT(EPOCH FROM now() - e.reported_at) / 604800)) AS raw_score FROM ward w JOIN event e ON ST_Contains(w.geom, e.geom) AND e.path = 'B' AND e.status <> 'resolved' JOIN class_weight c ON c.class = e.class GROUP BY w.ward_id, w.name;

## 3.4 Ward Sustainability Heatmap Scoring

Each ward receives a normalized 0 to 100 score from its open Path B items, weighted by class severity and age. Weights are configuration, not code, so ward offices can tune them.

| Band | Score | Colour | Operational meaning |
| --- | --- | --- | --- |
| Healthy | 0 to 20 | Green | Routine schedule |
| Watch | 21 to 40 | Light green / yellow | Monitor; include in next planned round |
| Elevated | 41 to 60 | Orange | Prioritize in next route |
| Critical | 61 to 100 | Red | Immediate crew dispatch; escalate to ward officer |

## 3.5 The Closed-Loop Feedback Cycle

| Step | Event | System response |
| --- | --- | --- |
| 1 | Municipal action update | Crew marks the item resolved with photo proof; status changes in PostGIS |
| 2 | Public dashboard refresh | Ward heatmap, scorecard and the citizen's tracker update automatically |
| 3 | Youth Dialogue Circle verification | Circle confirms on the ground, records the outcome in the Youth Action Log; only then is the item closed for good (or re-opened) |

## 3.6 Security, Privacy & Governance Controls

| Area | Control |
| --- | --- |
| Access | Role-based access (citizen, crew, PCR operator, ward officer, admin); PCR view limited to Path A items |
| Privacy | Edge inference; store detections and short evidence clips, not continuous video; retention limits set by the ward office |
| Integrity | Immutable event timestamps; every status change audit-logged |
| Transparency | Public dashboard shows ward-level counts and resolution status, not personal data |
| Model quality | Confidence thresholds, human review queue, and periodic retraining from verified outcomes |
| Technology stack | YOLO + OpenCV (vision), NLP models (complaint text), Python, Streamlit (dashboards), Pandas / NumPy (analytics), PostgreSQL / PostGIS (spatial database); live app and source are public |

# 4. UI/UX Wireframes & Interactive Dashboard Specifications

Four interfaces cover the full loop. Each is shown as a wireframe and as a text layout, followed by its component specification. All values in the wireframes are illustrative layout data, not measured results.

UX PRINCIPLES One-glance status (colour-coded, green to red), mobile-first citizen reporting in Hindi and English, accessible contrast and large touch targets, and no personal data on public screens.

## 4.1 Autonomous Rover Telemetry & Vision Feed

Figure 6. Wireframe 1: H.A.W.K. telemetry and live vision feed.

+-----------------------------------------+----------------------------------------------+ ¦ BLUEBRIDGE ¦ [Telemetry] Citizen Portal Ward Heatmap Youth Log ¦ +-----------------------------------------+----------------------------------------------+ ¦ LIVE VISION FEED (bounding boxes) ¦ ROVER STATUS: state ¦ battery ¦ range ¦ ¦ [litter .91] [person .88] [drain .84] ¦ CONTROLS: [STOP] [PATROL] [RETURN] ¦ +-----------------------------------------+----------------------------------------------+ ¦ DETECTION LOG: time ¦ class ¦ confidence ¦ path ¦ action ¦ +-----------------------------------------+----------------------------------------------+

| Component | Behaviour |
| --- | --- |
| Live vision feed | Streams the edge-node frame with bounding boxes and class + confidence labels |
| Rover status | Current state (STATE_*), battery, ultrasonic range, IR left/right, servo temperature, location tag |
| Controls | STOP (immediate fail-safe), PATROL, RETURN; commands authenticated and logged |
| Detection log | Time-ordered detections with the path taken (A to PCR, B to ward queue) |

## 4.2 Citizen Mobile / Web Omni-Channel Reporting Portal

Figure 7. Wireframe 2: citizen mobile app and web portal.

+--------------------------------+-------------------------------------------------------+ ¦ PHONE: Report an issue ¦ WEB: Web form ¦ App ¦ Helpline ¦ Rover auto-report ¦ +--------------------------------+-------------------------------------------------------+ ¦ [Garbage] [Drainage] [Safety] ¦ TRACK: Received>Classified>Crew>Resolved ¦ ¦ [photo + GPS pin] [text/voice] ¦ WARD MAP: my complaint + nearby open issues ¦ ¦ [ SUBMIT ] ¦ TRANSPARENCY BOARD: ward ¦ open ¦ resolved this week ¦ +--------------------------------+-------------------------------------------------------+

| Component | Behaviour |
| --- | --- |
| Issue picker | Garbage / waste, drainage / waterlogging, safety concern; free text or voice in Hindi or English |
| Photo + auto location | Camera capture with automatic GPS pin; manual pin adjustment |
| Complaint tracker | Received, AI-classified, crew assigned, resolved, with notifications |
| Transparency board | Ward-level open and resolved counts; no personal data |

## 4.3 Live GIS Ward Heatmap & Automated Triage Register

Figure 8. Wireframe 3: ward heatmap, scorecard and triage register.

+------------------------------------+---------------------------------------------------+ ¦ GIS WARD HEATMAP (green > red) ¦ SCORECARD: hazards, resolution time, alerts ¦ +------------------------------------+---------------------------------------------------+ ¦ [ ward grid coloured by score ] ¦ TRIAGE: ID ¦ class ¦ path ¦ conf ¦ status ¦ ¦ green = healthy, red = critical ¦ [Generate sanitation route] [Export ward report] ¦ +------------------------------------+---------------------------------------------------+

| Component | Behaviour |
| --- | --- |
| Ward heatmap | Green-to-red tiles or ward polygons from the scoring model; click to drill into items |
| Ward scorecard | Open hazards by class and age, average resolution time and trend, emergency alerts handled and pending |
| Triage register | Every AI decision with class, path, confidence and status; Path A rows highlighted; low-confidence rows marked Review |
| Actions | Generate priority sanitation route; export ward report for officials |

## 4.4 Youth-Led Peace & Dialogue Circles Action Log

Figure 9. Wireframe 4: Youth Action Log and the Dwividhta check.

+------------------------------------------+---------------------------------------------+ ¦ CIRCLE SUMMARY: circles held ¦ issues verified ¦ actions signed off ¦ volunteer hours ¦ +------------------------------------------+---------------------------------------------+ ¦ ACTION LOG: date¦ward¦action¦outcome ¦ DWIVIDHTA: AI flag + GIS + timestamp ¦ ¦ (rows of verified community actions) ¦ Circle verifies + explains; both close ¦ +------------------------------------------+---------------------------------------------+

| Component | Behaviour |
| --- | --- |
| Circle summary | Circles held, issues verified on the ground, actions signed off, volunteer hours |
| Action log | Date, ward and issue, action by the Youth Circle (site visit, cleanup, audit walk, awareness session), outcome |
| Dwividhta check | Shows both halves for each item: technology evidence and community verification; closure needs both |
| Community record | Feedback and consent notes; feeds the public dashboard refresh |

# 5. Scalability, Feasibility & Deployment Roadmap

## 5.1 Feasibility Snapshot

| LOW COST Arduino / ESP32 modular parts |  | OPEN SOURCE PostGIS + Python stack |  | WORKING Live app + public repo |
| --- | --- | --- | --- | --- |

## 5.2 Phased Implementation Blueprint (Months 1 to 12)

Figure 10. 12-month phased deployment roadmap.

| Phase | Months | Scope | Key deliverables | Exit criteria |
| --- | --- | --- | --- | --- |
| 1: Campus corridors | 1 to 4 | Managed campus corridors; controlled terrain and connectivity | Calibrated rover fleet; edge detector tuned; citizen portal live for the campus; PCR demo dashboard | Stable patrols; detection precision and recall recorded; Path A and Path B routing verified end to end |
| 2: Managed ward pilots | 5 to 8 | One or more managed wards with the ward office | Ward polygons loaded; heatmap and triage register in daily use; Youth Dialogue Circles started; sanitation route generation | Measured response-time change versus baseline; community adoption; false-alarm rate within agreed limit |
| 3: City-wide integration | 9 to 12 | Municipal and Police Control Room integration | Integration with municipal complaint systems and PCR workflows; multi-ward dashboards; operator training; public transparency board | Sustained operation across wards; municipal sign-off; documented SOPs |

## 5.3 Key Performance Indicators

| KPI | Target / measure | How it is measured |
| --- | --- | --- |
| Municipal cleanup time | 25 to 40% faster (projected target, to be validated in pilot) | Median time from report to resolution versus ward baseline |
| Emergency response time | 25 to 40% faster (projected target, to be validated in pilot) | Time from Path A alert to PCR acknowledgment and dispatch |
| Triage quality | Precision and recall per class, false-alarm rate | Labelled test set plus PCR verification outcomes |
| Citizen engagement | Reports per ward, repeat-reporter rate, tracker views | Portal analytics |
| Community verification | Share of closed items verified by Dialogue Circles | Youth Action Log |

## 5.4 Risk Analysis & Mitigation

| Risk | Likelihood | Impact | Mitigation strategy |
| --- | --- | --- | --- |
| Terrain obstacles and hardware wear | Medium | Medium | Restrict initial deployments to managed campus and ward corridors; rubber/plastic wheels; scheduled maintenance; modular spares |
| Public adoption friction | Medium | High | Pair rover patrols with Youth-Led Peace & Dialogue Circles to build trust; transparent public dashboard; simple bilingual reporting |
| False-positive emergency alerts | Medium | High | Confidence thresholds; human review queue; PCR verification before dispatch; retraining from false-positive logs |
| Edge data privacy | Medium | High | On-device inference; store only detections and short clips; role-based access; retention limits; no personal data on public screens |
| Connectivity gaps | Medium | Medium | Offline buffering on the rover; sync on reconnection; patrol-route planning around dead zones |
| Battery and thermal limits | Low | Medium | Low-voltage cut-off; servo duty-cycle limits; thermal soak testing; patrol length matched to battery |

## 5.5 Scalability Plan

| Layer | How it scales |
| --- | --- |
| Rovers | Add rovers per corridor; identical low-cost build; fleet view in telemetry tab |
| Edge vision | Inference stays on each rover, so cloud load does not grow with video volume |
| Database | PostGIS partitioned by ward; GIST spatial indexes; read replicas for dashboards |
| Dashboards | Role-based views per ward, zone and city; stateless Streamlit front end behind a load balancer |
| Governance | Youth Circles replicate ward by ward using a standard Action Log template |

# 6. Innovator Credentials & Verification References

## 6.1 Academic & Institutional Profile

| Name | Aravind K. T. |
| --- | --- |
| Programme | B.Tech in Electrical Engineering (Class of 2028) |
| Institution | Indian Institute of Technology Kanpur, Hall of Residence 5 |
| AISHE Code | U-0517 |
| State | Uttar Pradesh (project registration) |

## 6.2 National & International Accolades

| Recognition | Detail |
| --- | --- |
| Devbhoomi Rashtriya Ratna Puraskar 2025 | National Award for Outstanding Contribution to Youth Welfare |
| Youth Leadership Outstanding Awardee | Top 50 Youth Leaders in Asia-Pacific, Seoul |
| United Nations Social Forum 2025 | Officially accredited delegate, Palais des Nations, Geneva |
| i-SEA Fellow 2026 | Nanyang Technological University (NTU), Singapore |
| My Bharat Budget Quest 2026 | National Finalist, top among 13 lakh+ participants |
| Global Youth Festival 2026 (Paris) | Fully funded delegate selection |

## 6.3 Leadership, Certifications & Roles

| Role / credential | Detail |
| --- | --- |
| Former Secretary | Public Policy & Opinion Cell, IIT Kanpur |
| Founder | Youth-Led Peace & Dialogue Circles (Karnataka); the grassroots engine of the Dwividhta framework |
| UPG Sustainability Champion | Sustainability leadership recognition |
| UNESCO APCEIU GCED Certified | Global Citizenship Education certification |
| Official ICSC Ambassador | Ambassador role |

## 6.4 Official Verification Links

| Resource | URL |
| --- | --- |
| Digital portfolio (Google Sites) | https://sites.google.com/view/aravind-k-t-iitk/ |
| Live Streamlit web app | https://bluehawk-civic-nexus.streamlit.app |
| Public GitHub repository | https://github.com/kt2141/BlueHawk-Civic-Nexus |

# Annexure A: Document Notes

## A.1 Reference-Design Conventions

- Reference design items (pin allocation, firmware listing, rail values, schema, scoring bands and weights, test criteria) describe the baseline architecture and are meant to be reproducible and tunable.

- As-built wiring and code are in the public GitHub repository, which is authoritative where details differ.

- Dashboard values in wireframes are illustrative layout data. No measured outcomes are claimed in this document.

- The 25 to 40% improvement is a projected target to be validated in the managed pilot.

## A.2 Glossary

| Term | Meaning |
| --- | --- |
| H.A.W.K. | The autonomous edge-AI street patrol rover (hardware module) |
| BlueBridge | The AI/NLP classification and PostGIS GIS governance grid (software module) |
| Dwividhta | The Design Duality framework: technology precision plus community empathy |
| PCR | Police Control Room |
| Path A / Path B | Emergency-threat routing to the PCR / municipal-hazard routing to the ward heatmap and crews |
| PostGIS | Spatial extension for PostgreSQL used for geotagged indexing and ward queries |
| YOLO | Real-time object-detection model family used for bounding-box detection |
| MoYAS | Ministry of Youth Affairs & Sports |
| VBYLD | Viksit Bharat Young Leaders Dialogue (as referenced in the programme framework) |

## A.3 AI-Use Disclosure

The core classification engine uses YOLO/OpenCV (vision) and NLP models (text). Hardware design, Arduino firmware and governance frameworks are the innovator's original work. Generative AI was used to help draft and format this compendium; the innovator is responsible for its technical content.